SMART MANUFACTURING ASSIGNMENT

REPORT

Name: Krishna Agarwal

Roll No: 124ad0026

Subject: Compliant Mechanism Design

Mechanism: Foldable Phone Stand

Aim/Objective:

# To design an original compliant foldable phone stand mechanism using Python OCC, which uses flexible "living hinges" to create foldable motion through elastic deformation without traditional joints.

# Design Methodology

The design was created using Python OCC (Open CASCADE) library, which provides low-level 3D modeling capabilities. The process involved creating 3D primitives and combining them using Boolean operations.

- Step 1: Define Base Structure -  Created the main base plate using box primitive as the foundation.

- Step 2: Add Support Elements - Added back support and phone ledge using additional box primitives.

- Step 3: Combine Components - Used Boolean FUSE operations to join all parts together.

- Step 4: Create Compliant Features - Cut thin hinge sections using Boolean CUT operations to create flexible living hinges.

- Step 5: Add Functional Features - Created phone slot, finger holes, and rounded corners using CUT operations.

- Step 6: Export Final Model - Exported the compliant mechanism as CAD.stp file for manufacturing.

# Working Principle

# When the back support is folded upward, the thin compliant hinge sections (1mm thickness) bend elastically, allowing the structure to transform from flat storage position to functional stand position. The elastic deformation of the living hinge material provides the necessary flexibility while maintaining structural integrity. When unfolded, the material returns to its original shape due to its elastic properties.

# Applications

- Portable device accessories - Phone and tablet stands for mobile use.

- Desktop organizers - Foldable stands for offices and workspaces,

- Travel accessories - Compact solutions for travelers,

- Consumer products - Affordable, mass-producible stands,

- 3D printed accessories - Customizable stands for different devices.

# Advantages of Compliant Mechanisms

- Single-piece construction - No assembly required, reduced manufacturing costs.

- Wear-free operation - No mechanical joints to wear out over time.

- Compact storage - Folds completely flat when not in use.

- High reliability - Fewer failure points than traditional mechanisms.

- Easy manufacturing - Suitable for 3D printing, injection molding, or laser cutting.

# Learning Outcomes

- Understanding Python OCC - Learned to use Open CASCADE technology for 3D modeling

- Primitive operations - Gained experience with box and cylinder primitives as building blocks

- Boolean operations - Mastered FUSE and CUT operations for complex geometry creation

- Compliant design principles - Understood how to design flexible mechanisms using living hinges

- CAD export skills - Learned to export designs to industry-standard STEP format

- Parametric thinking - Developed approach to create modifiable, scalable designs

# Conclusion

Using Python OCC provides a powerful, programmatic approach to designing compliant mechanisms. The code-based methodology allows for precise control over geometric features and enables easy modification of design parameters. The foldable phone stand demonstrates how compliant mechanisms can replace traditional hinged designs with simpler, more reliable single-piece constructions that leverage material flexibility for motion.